# NanoPi R5S + OpenClash Practical Deployment & Troubleshooting Guide

This document summarizes a real-world home lab deployment experience using NanoPi R5S, FriendlyWrt, and OpenClash. The goal is long-term stability instead of chasing benchmark numbers. It avoids version-specific UI screenshots and focuses on architecture, troubleshooting logic, and operational experience.

## 1. Recommended Network Architecture

Recommended structure: ISP modem → Main Router → NanoPi R5S (bypass router) → Clients/APs.

Avoid using NanoPi R5S as the primary PPPoE router unless you fully understand OpenWrt networking.

The most stable approach is usually: main router handles core networking, bypass router handles proxy/routing.

Apple TV and streaming devices should ideally use wired Ethernet.

## 2. Initial Setup

Use a stable FriendlyWrt release instead of experimental builds.

Change the default password immediately after first login.

Verify system time is correct before installing proxy services.

Before installing OpenClash, confirm that local internet access works normally.

## 3. Bypass Router Mode

One of the most common mistakes is DHCP conflict.

If the main router already provides DHCP, disable DHCP on NanoPi R5S.

Suggested example:

Main router: 192.168.1.1

NanoPi R5S: 192.168.1.2

This avoids unstable IP allocation problems.

## 4. Correct OpenClash Workflow

Do not start tweaking advanced settings immediately.

Recommended order:

1) Verify OpenClash core works

2) Verify subscription updates

3) Verify nodes connect correctly

4) Verify DNS

5) Optimize rules only after everything becomes stable

## 5. DNS Lessons Learned

Many streaming problems are actually DNS-related rather than bandwidth-related.

Typical symptoms included:

- Apple TV+ buffering despite high speed tests

- YouTube homepage loading but videos failing

- Intermittent website failures

The biggest lesson: avoid running too many DNS systems simultaneously.

## 6. TUN Mode Experience

TUN mode is powerful but can introduce instability.

Earlier issues included random connectivity loss and LAN access problems.

It is better to stabilize the basic proxy logic before enabling complex TUN features.

## 7. IPv6 Reality

IPv6 is theoretically excellent, but ISP implementation quality varies.

Disabling IPv6 actually improved stability in some real-world scenarios.

If unexplained instability appears, IPv6 should be one of the first suspects.

## 8. Node Switching Mistakes

One major mistake was switching nodes too frequently.

This made troubleshooting impossible because too many variables changed simultaneously.

Final rule learned: change only one variable at a time.

## 9. Apple TV Streaming Lessons

Bandwidth alone does not determine streaming quality.

Routing quality, packet loss, DNS behavior, and peak-hour congestion all matter.

Speed tests are not always representative of real streaming experience.

## 10. Long-Term Stability Philosophy

Do not upgrade firmware too frequently.

Keep backup configurations.

Stable systems are often more valuable than highly optimized but fragile systems.